The "Rubidium PC"

To synthesize a desired computer clock timing frequency the

accompanying circuit has an output frequency that is a rational

multiple of an available standard input frequncy.  Two counters

determine the frequency ratio.  They are set to divide the

standard frequeny by M and the output frequency by N.  A phase

lock loop regulates the output frequency such that

(standard frequency)/M = (output frequency)/N = F

The phase detector operating at frequency F controls the output

frequency, which is generated by a voltage controlled oscillator

(VCO).

In the schematic, counters U3, U4, U5, U6 divide the VCO

frequency by N.  Dip switches SW1 and SW2 are set to -N.

Similarly U8, U9, U10, U11 divide the reference standard

input frequency by M; switches SW3 and SW4 are set to -M.

Phase lock condition can be checked with a DC voltmeter at U2 pin

5 or at C5.  If either the reference or VCO signal is absent, the

control voltage will remain near ground or power supply level.

If signals are present but not locked, the average DC level will

be near mid-supply.  When locked, the level will change with

changes in capacitance of the crystal circuit -- for example by

tuning C3.

Best actual values of R1, C1, C2 to use depend on frequency and

crystal characteristics but are not very critical.  The time

constant R4,C5 is not critical but it should increase as the

phase detector frequency F decreases.  The values shown were used

for F = 200KHz approximately.

D1 is a variable capacitance diode that tunes the crystal

oscillator frequency. An ordinary computer diode or transistor

junction also exhibits voltage-variable capacitance, but only in

small degree.

REF INPUT is from a standard frequency generator such as

a rubidium-stabilized oscillator.

Although 16-bit counters are shown, a ratio of small numbers

such as 10MHz * 63/44 = 14.318181818...MHz needs only 8-bit

counters.

S. L. Moshier

March, 1998